B.TECH/ECE/6TH SEM/ ECEN 3202/2019

DIGITAL SIGNAL PROCESSING & APPLICATIONS

(ECEN 3202)

Time Allotted : 3 hrs						      Full Marks : 70

Figures out of the right margin indicate full marks.

Candidates are required to answer Group A and

any 5 (five) from Group B to E, taking at least one from each group.

Candidates are required to give answer in their own words as far as practicable.

Group – A

(Multiple Choice Type Questions)

1. 	Choose the correct alternative for the following: 	10 × 1 = 10

- The period of the signal x(n) = cos( π/2 n) cos(π/4 n) is

- 8		(b) 4 		(c) 32  	(d) 12.

- The ROC of a finite duration discrete time signal is as follows

(a)	Entire z-plane except at z=0       	 (b) Entire z-plane except at z=∞

(c) Entire z-plane                            	 (d) A ring in the z-plane.

(iii) 	The N point DFT of finite length signal  x(n) = δ (n) is

(a) 0  	(b) WN 	(c) z-1	(d) 1.

(iv) 	Direct evaluation of N point  DFT require

(a) N2 multiplication and N2 addition

(b) N multiplication and N addition

(c) N(N-1) multiplication and N2 addition

(d) N2 multiplication and N(N-1) addition.

(v) 	The convolution of u(n) and u(n-4) at n=5 is

(a) 2	 (b) 1	(c) 5	(d) 0.

(vi) 	In the bilinear transformation method, the relationship between the analog frequency, Ω and digital frequency, ω is given by

.

(vii) 	The poles of a Butterworth filter lie on a/an

(a) circle 	(b) parabola	(c) ellipse 	(d) helix.

(viii)	 FIR filter is always stable because all of its poles are ___________

(a) at the origin 	(b) at the infinity

(c) at the ROC  	(d) none of these.

(ix) 	A baseband signal has a spectral range that extends from 20 Hz to 82 KHz. The acceptable range of the sampling frequency is

(a) >40 Hz,<82 KHz                         	 (b) 40 Hz to 82 KHz

(c)  ≥164 KHz                                   	 (d) ≤164 KHz.

(x)	For a rectangular window of N sample , width of main lobe is

(a) 2π/N 	(b) 4 π/N  	(c) 3 π/N	 (d) π/N.

Group – B

2.  (a) 	Determine the pole zero plot for the system described by difference equation y(n) – ¾ y(n-1) + 1/8 y(n-2)  =  x(n)  –   x(n-1).

Check the system whether it is stable or unstable for |z| >1/2.

- Let y(n) denote the convolution of h(n) and g(n), where, h(n) = (1/2)n u(n) and g(n)  is causal sequence . If y(0) = 1 and y(1) =1/2 then find the value of g(1).

- Determine the following system is time variant or time invariant.

y(n)  =  x(-n)

5+ 5 + 2 =12

3. (a)	An LTI system is characterized by the system function

Determine h(n) if the ROC is:

(b)	Determine the zero-input response and zero-state response of the system characterized by the difference equation

6 + 6 = 12

Group – C

4. (a)	Perform the circular convolution of the following two sequences using graphical method

x1(n)={1,2,2,3,4,}  ↑    x2(n)={2,3,1,1,2}   ↑.

(b)	A length-8 sequence is given by x(n)={-4, 5, 2,-3,0,-2, 3, 4}↑                  , 0≤n≤7, with

an 8-point DFT given by X(k). Determine the sequence y(n) whose 8-point DFT is given by yk=e-j2π3k4 xk.

(c)	What do you mean by zero-padding?

6 + 5 + 1= 12

5. (a) 	Determine the N point IDFT of X(k) = δ(k)  0 ≤ k ≤ N-1 and hence for                  0 ≤ k0 ≤ N-1 , determine the DFT of x1(n) = cos(2 π k0 n/N).

(b)	Find the 8 point DFT of sequence x(n) = {1,1,1,1,0,0,0,0} using radix 2 DIF-FFT algorithm.

4 + 8 =12

Group – D

6. (a)	Compare between IIR digital filter and FIR digital filter.

(b)	A digital Butterworth filter has to be designed using bilinear transformation. The filter specifications are as follows:

0≤w≤0.2π

0.6π≤w≤π

Find the filter order N and cut-off frequency.

(c)	Explain the necessity of windowing method in FIR filters.

(d)	State the disadvantage of impulse invariant technique.

3 + 4 +3 +2 = 12

7. (a)	Evaluate the order and the poles of a Butterworth filter which has 2dB attenuation at 1000Hz and 15dB attenuation at 2000Hz. Determine the system function H(z) by bilinear transformation using .

(b)	Obtain the digital filter transfer function H(z) from its analog counterpart with transfer function Ha(s) using impulse-invariant method.

8 + 4 = 12

Group – E

8. (a)	Obtain the Direct Form-I and Direct Form-II realization for a third order IIR transfer function which is expressed as below:

(b)	Determine a cascade realization of the system characterized by the transfer function which is expressed as under:

(4 + 4) + 4= 12

9. 	Write short notes on any two of following.

- Fourier series method for FIR filter.

- Design of Butterworth filter.

- Application of Multi-rate Digital Signal Processing.

- Architecture of TMS 320C5416.

(6  2) =12